ESP32-SETUP (NOTES)

Configuration:

- I2S0 = Master  (Pair 1)

- I2S1 = Slave   (Pair 2)

Pair 1 generates the clock and Pair 2 consumes it

PCB Wiring:

Pair 1 (I2S0 Master):

SCK → GPIO 26 (I2S0 BCLK — master clock output)

WS  → GPIO 25 (I2S0 WS   — master WS output)

SD  → GPIO 27 (I2S0 DATA)

Pair 2 (I2S1 Slave):

SCK → GPIO 32 (I2S1 BCLK — slave clock INPUT, wire to GPIO 26)

WS  → GPIO 14 (I2S1 WS   — slave WS INPUT,   wire to GPIO 25)

SD  → GPIO 33 (I2S1 DATA)

In our PCB The Following Pins will be bridged together

GPIO26 ─── GPIO32   (shared BCLK)

GPIO25 ─── GPIO14   (shared WS)

L/R Selection (both pairs):

Left  mic: LR pin → GND

Right mic: LR pin → VDD (3.3V)

CONFIG pins: All tied to GND

Data format:

DMM-4026-B-I2S-R outputs 24-bit audio in MSB of 32-bit Philips I2S word.

Lower 8 bits are always zero. Right-shift by 8 to get signed 24-bit value.

Clock:

48000 Hz sample rate × 64 bits per frame = 3.072 MHz BCLK